Creative Stage Lighting Co., Inc.

P.O. Box 567

Route 28N

North Creek, NY  USA 12853

Tel: 518.251.3302

Fax:518.251.2908  24 Hours

www.creativestagelighting.com

PRODUCT SPECIFICATION

DURA-FLEX Part Number: 12/142586

Product Description :                                                                                                                                                                                       	A Twelve (12) Gauge --  Fourteen (14) Conductor -- Bare Copper With PVC Insulation, Reverse Planetary Laid, Rugged, Oil Resistant, PVC Jacketed, Yet Highly Flexible Instrumentation Cable.

Conductor Data:

Material				Bare Copper

Gauge				12 AWG

Stranding			65 x 30

Insulation Data:

Material				Polyvinylchloride

Nominal  Wall Thickness		.022”

Diameter (primary)		.140”

Cabling Data:

Number Of Conductors		14

Conductor Identification		Cond. 1		Red, No Number

Cond. 2 - 12	White, Black Number

Cond. 13	Green

Cond. 14	Green With Yellow Stripe

Center Filler			Polyvinylchloride / Black

O/A				Tissue Tape

Jacket Data:

Material				Polyvinylchloride (Oil Resistant)

Color 				Black

Markings			White Lettering, Also Indented

Nominal Wall Thickness		.080”

Diameter (O.D.)			.785”

Legend				DURA-FLEX 12AWG 14/C RU AWM STYLE 2586 600V 105C VW-1 E167231-A

---- CSA AWM  I A/B  II A/B 105C 600V FT1 LL71737

Product Ratings:

Weight				.535 lbs./ft.

Temperature Range                        -20°C to 105°C

Voltage   			600 Volts

UL Approval			RU AWM STYLE 2586 600V 105C

CSA Approval			CSA AWM  IA/B   IIA/B 105C 600V FTI